SMARTPHONE-BASED DETECTION OF DYES IN WATER FOR ENVIRONMENTAL SUSTAINABILITY (2016)

The problem the study is addressing to is dyes in the wastewater discharging into the environment, which is very harmful for humans and also the ecosystem. Industrial dyes are a health hazard for humans also can contaminate water bodies. The use of adsorption is studied to remove dye molecules from the wastewater.

The motivation is to develop smartphone-based platform that can replace traditional devices used to solve this problem like colorimeters and spectrometers. The solution the study is offering is user friendly, ultra-low cost and giving compatible performance while contributing to environmental sustainability. Cost-effective solution, use of smartphone technology and only 3D printer, environment-friendliness and innovative approach.

The study is novel in that it fully integrates plastic optical fibers, allowing fort he collection of the light from the smartphone’s flash. Allowing for on-site analysis, real-time monitoring and management of water quality in environmental settings.

This paper is introducing an ultra-low cost and portable smartphone-based platform designed for spectrometric and colorimetric analysis. The main aim is to investigate the use of electrospun polyacrylonitrile (PAN)/zeolite nanofibers for the adsorption of dyes, with a focus on their potential for field-deployable environmental and wastewater management.

PAN and Zeolite is used to combine their advantages. Methylene blue (MB) is used as a dye in the Project. When MB molecules are adsorbed onto the large areas of nanofibers they reduce the absorbance of the liquid containing the dye. To measure the absorbance and quantify the adsorption process the study offers a solution by designing a custom cradle aligning plastic optic fibers with the flashlight of a smartphone. The study achieves high-resolution measurements using only 3D printed polymer parts meaning the study overcame the need for external emissive sources, lenses, electrical sources.

The study is offering a smartphone-based adsorption solution for wastewater management for environmental sustainability in a cost-effective, robust, and user-friendly manner.

SUMMARY OF THE EXPERIMENT

Cradle, smartphone holder, cuvette holders were 3D printed with ABS filament. The system has 2 PMMA fibers. 1.5 mm diameter fiber couples the light from the cellphone flash to the cuvette and a 0.25 mm fiber collects the transmitted light to the smartphone camera. RAW images are used to overcome the JPEG format’s disadvantages. Smartphones cannot process RAW data images, in this study .DNG images, Images were transferred to a computer to be processed in MATLAB. All the images used for dataset were captured in the same environmental conditions. From .DNG, the images were converted to TIFF for easier extraction of RGB values (DCRAW software was used to convert b/w formats). RGB values were transformed to HSV due to HSV being more robust to illumination variation.

SMARTPHONE-BASED COLORIMETRIC DETECTION VIA

MACHINE LEARNING (2017)